### FAB-SDN Fabric Setup Mapping Table ##### 1. Document Overview and Scope This technical reference defines the FAB-SDN functional group, which encompasses the infrastructure and protocols required to deploy Software-Defined Networking (SDN) fabrics. This document is designed for Lead Network Architects and Multi-Vendor Specialists transitioning between or managing environments that utilize Cisco Nexus (NX-OS) and Extreme Networks (Switch Engine/Fabric Engine) platforms. The scope of this reference includes: * SDN Underlays: Physical reachability and core routing, with a specific focus on IS-IS as the primary link-state protocol. * Fabric Overlays: Virtualized control planes, specifically BGP EVPN implementations. * Encapsulation Mapping: Functional translation between VXLAN/VNI methodologies and IS-IS/SPBM/I-SID architectures. ##### 2. Multi-Vendor Fabric Mapping Table | Functional Category | Cisco NX-OS (Programmability Focus) | Extreme Switch Engine (EXOS) | Extreme Fabric Engine (VOSS) | Other Multi-Vendor (IOS-XE/SLX-OS) | Architect’s Note | | ------ | ------ | ------ | ------ | ------ | ------ | | Fabric Infrastructure Initialization | Enable NETCONF , RESTCONF , or gRPC agents for DME access. | enable stacking-support | Transition Universal Hardware to Fabric Engine mode. | Information Not in Source | Switch Engine requires stacking-support enabled (p. 1434) to facilitate fabric-ready universal hardware states. | | Underlay Routing (Core Connectivity) | Utilization of Native (Device) YANG Models to define L3 reachability. | create isis area | IS-IS (Backbone Link) | Information Not in Source | Cisco NX-OS manages the underlay via the DME database; Extreme uses explicit IS-IS area creation (p. 1721). | | Overlay Control Plane (LISP/EVPN) | gRPC / YANG configuration of L2VPN EVPN namespaces. | create bgp evpn instance | SPBM Control Plane | Information Not in Source | Extreme Switch Engine maps EVPN instances (p. 1694) to handle the macro-segmentation control plane. | | Data Plane Encapsulation | VXLAN data plane representation via Common (OpenConfig) YANG Models . | create virtual-network | SPBM / IEEE 802.1aq | Information Not in Source | Functional parity: Cisco's VXLAN construct is mirrored by Switch Engine's virtual-network (p. 1770). | | Service Mapping (VNI / I-SID) | Binding VLANs to VNIs within the DME Model Infrastructure. | configure vlan add nsi | isid  | configure fabric attach isid-nsi-offset | Information Not in Source | | Fabric Attachment & Edge Setup | Model-driven automation of edge port telemetry and state. | configure fabric attach ports | Fabric Attach (FA) Server/Proxy | Information Not in Source | Fabric Attach (p. 538) automates the mapping of edge devices to fabric services ( I-SIDs ). | ##### 3. Fabric Setup Logic & Encapsulation Comparison * Cisco SDN Approach (NX-OS): * Relies on a Model-Driven Programmability ( MDP ) framework where the DME (Data Management Engine) database serves as the single point of truth. * This architecture ensures consistency across CLI , NETCONF , RESTCONF , and gRPC interfaces. * Logic is centered on YANG data modeling (Native and OpenConfig) to define how VXLAN/VNI segments are instantiated and monitored via observability agents. * Extreme Fabric Approach (Switch Engine/Fabric Engine): * Leverages Universal Hardware , which allows a single physical switch to run either Switch Engine (formerly EXOS) or Fabric Engine (formerly VOSS) as of Release 31.6. * The methodology utilizes IS-IS and Shortest Path Bridging (SPBM) for the fabric underlay/control plane. * Service identification is managed through I-SID (I-Component Service Identifier) or NSI (Network Service Identifier). * Fabric Attach (FA) logic (p. 534) is used to extend fabric services to the edge, utilizing specific offsets for I-SID-to-NSI mapping to ensure seamless multi-vendor attachment. ##### 4. Verification and Observability Commands | Functional Verification Task | Command (Switch Engine Focus) | | ------ | ------ | | Show BGP EVPN details | show bgp evpn | | Show BFD session status | show bfd | | Show CDP neighbor info | show cdp neighbor | | Show CFM connectivity details | show cfm | | Show centralized database status | show database | | Show Virtual Router counters | show counters vr | | Clear Virtual Network counters | clear counters virtual-network | | Clear IS-IS protocol counters | clear isis counters | | Clear BGP flap statistics | clear bgp flap-statistics | -------------------------------------------------------------------------------- Architect's Final Note: All Cisco NX-OS configurations in this document focus on the Programmability Agent and Model Infrastructure as defined in the Release 10.5(x) context. For Switch Engine, ensure proper Access Levels (Administrator) are used before executing create or configure operations.