### SEC-ID Functional Group: Identity and Authentication Command Mapping Table ##### 1. Document Overview and Scope This document serves as a senior-level technical reference and cross-platform mapping table for the SEC-ID functional group. It focuses on the architectural and command-level implementation of Identity Management, AAA (Authentication, Authorization, and Accounting), and Policy-based Authentication. This guide is designed for network security architects to facilitate the migration, integration, and management of diverse network environments. The mapping covers the following five target platforms: * Cisco IOS-XE * Cisco NX-OS * Extreme Switch Engine (EXOS) * Extreme Fabric Engine (VOSS) * Extreme SLX-OS Data presented herein is derived from authoritative source documentation. Users should note the rebranding of Extreme Networks operating systems as of Release 31.6: ExtremeXOS (EXOS) is now Switch Engine , and VSP Operating System Software (VOSS) is now Fabric Engine . -------------------------------------------------------------------------------- ##### 2. SEC-ID Command Mapping Table The following table maps configuration commands and programmable interface requirements across the supported platforms. For Cisco NX-OS, configuration is performed through Programmable Interface Agents using YANG models rather than traditional CLI in this functional context. | Cisco IOS-XE | Cisco NX-OS | Extreme Switch Engine (EXOS) | Extreme Fabric Engine (VOSS) | Extreme SLX-OS | | ------ | ------ | ------ | ------ | ------ | | Local Account & Password Policy | Configuration via NETCONF, RESTCONF, or gRPC Agents using YANG models. | configure account

configure account password-policyclear account lockoutcreate accountdelete accountdisable accountenable account | Information not available in source context | Information not available in source context | | AAA Server Configuration (RADIUS/TACACS) | Configuration via NETCONF, RESTCONF, or gRPC Agents. Model Infrastructure handles protocol initiation for authentication and handshaking. | configure radiusconfigure radius shared-secretconfigure radius tls ocspconfigure radius-accounting shared-secretconfigure tacacsconfigure tacacs timeoutconfigure tacacs-accounting shared-secretconfigure ldap domaincreate ldap domaindisable radiusdisable tacacsenable radiusenable tacacs | Information not available in source context | Information not available in source context | | Network Login & Dot1x | Configuration via NETCONF, RESTCONF, or gRPC Agents using YANG models. | configure netloginconfigure netlogin dot1xconfigure netlogin dot1x eapol-transmit-versionenable netlogindisable netloginclear netlogin stateclear netlogin state agent | Information not available in source context | Information not available in source context | | Identity Management & Role-Based Access | Configuration via NETCONF, RESTCONF, or gRPC Agents using YANG models. | configure identity-managementconfigure identity-management detectionconfigure identity-management kerberos snoopingcreate identity-management rolerefresh identity-management roledisable identity-managementenable identity-management | Information not available in source context | Information not available in source context | | Hardware & Interface Security | Configuration via NETCONF, RESTCONF, or gRPC Agents using YANG models. | configure macsecclear macsec countersconfigure mac-lockdown-timeoutenable mac-lockingdisable mac-lockingconfigure inline-powerclear inline-power stats portscreate macsec connectivity-association | Information not available in source context | Information not available in source context | -------------------------------------------------------------------------------- ##### 3. Hardware and Status Verification Commands Operational status and observability telemetry are mapped below to ensure consistent troubleshooting and auditing. | Cisco IOS-XE | Cisco NX-OS | Extreme Switch Engine (EXOS) | Extreme Fabric Engine (VOSS) | Extreme SLX-OS | | ------ | ------ | ------ | ------ | ------ | | Information not available in source context | Observability Agents:NETCONF (SSH/XML)gRPC (HTTP/Protobuf)Telemetry (HTTP/UDP/JSON) | show identity-managementshow identity-management detectionshow netloginshow accounts

show accounts password-policyshow radiusshow radius-accountingshow tacacs | Information not available in source context | Information not available in source context | -------------------------------------------------------------------------------- ##### 4. Technical Usage Notes and Constraints The following guidelines and architectural constraints are derived from platform specifications: * Switch Engine Object Naming Rules: * Uniqueness: Object names must be unique within their respective configuration category. * Composition: Names must begin with an alphabetical character. They may contain alphanumeric characters and underscores ( _ ). Spaces are prohibited. * Length: Maximum string length is 32 characters. * Reserved Keywords: Keywords such as vlan or STP (and other second-level keywords) are reserved and cannot be used as primary object names. * Switch Engine Character Restrictions: * The characters & (ampersand), < (left angle bracket), and > (right angle bracket) are strictly prohibited in the CLI as they are reserved for XML processing. * Cisco NX-OS Programmable Interface Specifications: * NETCONF: Utilizes SSH transport with XML encoding. * RESTCONF: Utilizes HTTP transport with XML or JSON encoding. Implementation is based on draft-ietf-netconf-restconf-10. * gRPC: Utilizes HTTP transport with Google Protobuf encoding. * Model Infrastructure: NX-OS maintains a centralized DME database. Both Native (Device) and Common (OpenConfig) YANG models serve as wrapping representations of this database, ensuring consistency across all agents. * Operational Completion: * Switch Engine utilizes a "namespace" for lengthy values (such as stack node addresses) to support Tab-based command completion. -------------------------------------------------------------------------------- ##### 5. Platform Version and Compatibility Reference Minimum software requirements for Identity and Power over Ethernet (PoE) features are summarized below for the Extreme Switch Engine series. | Switch Series | Switch Models | Minimum Software Version Required | | ------ | ------ | ------ | | 4120 | 24MW-4Y, 48MW-4Y | Switch Engine 32.7.1 and later | | 4220 | 12P, 24P, 48P, 4MW-8P, 4MW-20P, 8MW-40P | Switch Engine 32.7.1 and later | | 5320 | 48P-8XE, 24P-8XE | Switch Engine 31.6 and later | | 5320 | 16P-4XE, 16P-4XE-DC | Switch Engine 31.7 and later | | 5420 | 5420F and 5420M PoE models | ExtremeXOS 31.3 and later | | 5520 | 24W, 48W, 12MW-36W | ExtremeXOS 31.1 and later | | 5720 | 24MW, 24MXW, 48MW, 48MXW | Switch Engine 32.1 and later | | 7520 / 7720 | 48Y, 48XT, 32C | Switch Engine 32.7.1 and later | Rebranding Context: As of Release 31.6, ExtremeXOS (EXOS) has transitioned to the name Switch Engine , and VOSS has transitioned to Fabric Engine . All historical documentation references to ExtremeXOS are applicable to Switch Engine 31.6 and subsequent releases.