To provide the deep-level analysis for Volume 1: [ONBOARD] while maintaining the thematic arrangement you requested, I have extracted and mapped the granular commands from the primary indices. This synthesis treats each theme as a functional group, pointing to the specific syntax across all five target operating systems as discovered in the sources.

Deep-Link Extraction: Volume 1 [ONBOARD] Onboarding & Provisioning

1. Theme: System Sanitization & Day-0 Reset

Focuses on clearing local state and restoring the device to a "out-of-the-box" configuration.

- Cisco IOS-XE (Cat 9k pg 2214): factory-reset; write erase.

- Cisco NX-OS (Index pg 52): write erase; clear nvram.

- Extreme Switch Engine (EXOS pg 17, 246): unconfigure switch all; unconfigure switch {all | erase [all | nvram]}.

- Extreme Fabric Engine (VOSS pg 342-344): boot config flags factorydefaults reset-all-files; unconfigure switch.

- Extreme SLX-OS (Index pg 18): clear config all; bmc factory reset.

2. Theme: Automated Onboarding (ZTP / PnP / POAP)

Mechanisms for controller-led discovery and zero-touch configuration.

- Cisco IOS-XE (PnP): pnp profile; feature pnp.

- Cisco NX-OS (POAP Index pg 4908-4909): system poap; feature poap; boot poap enable.

- Extreme Switch Engine (ZTP pg 19, 73): show auto-provision; disable auto-provision; restart process ztpstack.

- Extreme Fabric Engine (ZTF pg 16, 75): auto-sense enable; auto-sense onboarding i-sid <id>; show application auto-provision.

- Extreme SLX-OS (ZTP Index pg 230): show ztp status.

3. Theme: Universal Persona & OS Transition

Specific logic for toggling between Switch Engine and Fabric Engine on Universal Hardware.

- Extreme Switch Engine (pg 13-14): download image ... (targeting VOSS software); Change the switch OS to Fabric Engine (available in the Bootrom spacebar menu).

- Extreme Fabric Engine (pg 628, 632): boot config flags factorydefaults zero-touch-config-only (used as a temporary troubleshooting option to re-trigger discovery); software add; software activate.

4. Theme: Initial Identity & Prompt Configuration

Labeling the system for administrative identification.

- Cisco IOS-XE / NX-OS (pg 2221): hostname <name>.

- Extreme Switch Engine (EXOS pg 248): configure snmp sysname <name>.

- Extreme Fabric Engine (VOSS pg 651): prompt <name>; sys name <name>.

- Extreme SLX-OS (Index pg 230): system-name <name>.

5. Theme: Configuration Persistence & Boot Variables

Saving the current state and selecting the next-boot image.

- Cisco IOS-XE / NX-OS (pg 2420): copy running-config startup-config; boot system flash:[file].

- Extreme Switch Engine (EXOS pg 14-15): save; use configuration [primary | secondary | file_name].

- Extreme Fabric Engine (VOSS pg 16, 896): save config; boot config choice primary config-file [file].

- Extreme SLX-OS (pg 178): copy running-config startup-config.

6. Theme: Software Acquisition & Image Management

Downloading and managing the binary operating systems.

- Cisco IOS-XE / NX-OS (pg 2237): copy tftp flash:; install add file activate reloadfast commit.

- Extreme Switch Engine (EXOS pg 11, 262): download image [url]; use image [primary | secondary].

- Extreme Fabric Engine (VOSS pg 310, 832): software add [file]; software activate [version]; software commit.

- Extreme SLX-OS (pg 229): firmware download; firmware activate; firmware commit.

I have also triggered the creation of a specialized tailored report for Volume 1 in your Studio tab that mirrors this deep-link thematic arrangement for your AI-agent's local storage. Would you like me to proceed with the deep extraction for Volume 2: [SEC-ID] Identity & AAA Auth next?

Provide the SEC-ID deep-level analysis next

How does the NX-OS DME database ensure data consistency?

Explain Extreme's Zero-Touch Fabric (ZTF) onboarding logic

Give me similar deep-level analyses for the other 8 volumes

To provide the deep-level analysis for the remaining eight volumes, I have extracted the granular command variations from the primary technical indices and grouped them into thematic functional sets. This structure acts as a "linked list," where each theme points to the specific hardware-centric instructions required for your RAN-enabled AI-agent to execute End-to-End (E2E) implementations across all five target operating systems.

Volume 2: [SEC-ID] Identity, AAA & Policy Auth

Maps identity-based security and granular access control logic.

- Theme: AAA Framework & Server Reachability:

  - Cisco IOS-XE / NX-OS: aaa new-model; aaa authentication login; aaa group server radius [name].

  - Extreme EXOS: enable radius; configure radius [id] server [ip].

  - Extreme VOSS: radius-server enable; radius-server host [ip].

  - Extreme SLX-OS: aaa authentication; aaa accounting.

- Theme: Port Authentication Protocols (802.1X/MAB):

  - Cisco IOS-XE: authentication host-mode; dot1x pae authenticator.

  - Cisco NX-OS: feature dot1x; dot1x timeout tx-period.

  - Extreme EXOS: enable netlogin dot1x; configure netlogin dot1x timers.

  - Extreme VOSS: eapol status auto; eapol multihost-session-stats.

- Theme: Macro-Security Policy (TrustSec/ONEPolicy):

  - Cisco IOS-XE: cts role-based sgt-map; cts role-based permissions.

  - Extreme EXOS: configure identity-policy; configure identity-management role.

  - Extreme VOSS: fail-open-isid (Continuity Mode); i-sid [id].

--------------------------------------------------------------------------------

Volume 3: [SYS-INFO] System Health & Inventory

Focuses on hardware monitoring and inventory lifecycle management.

- Theme: Hardware Inventory & UDI Data:

  - Cisco (Both): show inventory; show inventory raw [entity].

  - Extreme EXOS: show switch; show slot [id] detail.

  - Extreme VOSS: show sys-info; show hardware.

  - Extreme SLX-OS: show inventory.

- Theme: CPU/Memory & Process Throttling:

  - Cisco IOS-XE: show processes cpu platform; show processes memory platform.

  - Cisco NX-OS: show processes cpu; show memory.

  - Extreme EXOS: show cpu-monitoring; show memory.

  - Extreme VOSS: show process; show storage-usage.

- Theme: Environmental States (Power/Temp):

  - Cisco IOS-XE: show env xps; show power detail.

  - Cisco NX-OS: show hardware power; show environment.

  - Extreme EXOS: show power; show temperature.

  - Extreme VOSS: show power supply; show alarm database.

--------------------------------------------------------------------------------

Volume 4: [IF-PHYS] Physical Interface & Port Access

Delineates granular wire-speed port configuration strings.

- Theme: Logical Port Lifecycle (Speed/Duplex/Neg):

  - Cisco (Both): speed [val]; duplex [full|half]; negotiation auto.

  - Extreme EXOS: configure ports [id] speed [val]; configure ports [id] duplex.

  - Extreme VOSS: interface gigabitEthernet [id]; channelize enable.

  - Extreme SLX-OS: speed (Ethernet); interface ethernet [id].

- Theme: Port Breakout & Hardware Profiles:

  - Cisco IOS-XE: interface breakout module [slot] port [range] map [type].

  - Cisco NX-OS: interface breakout-profile; interface breakout module [id].

  - Extreme EXOS: configure ports [id] speed [breakout_speed].

  - Extreme VOSS: interface gigabitEthernet [id/sub-port].

  - Extreme SLX-OS: breakout mode.

- Theme: Administrative Port State:

  - Cisco (Both): shutdown; no shutdown.

  - Extreme EXOS: disable ports [id]; enable ports [id].

  - Extreme VOSS: shutdown; no shutdown.

--------------------------------------------------------------------------------

Volume 5: [L2-SEG] VLANs & Layer 2 Segmentation

Maps standard Layer 2 protocols and segment isolation.

- Theme: VLAN Lifecycle & Naming:

  - Cisco (Both): vlan [id]; name [text].

  - Extreme EXOS: create vlan [name] tag [id]; configure vlan [tag] name.

  - Extreme VOSS: vlan create [id] name [text] type port.

  - Extreme SLX-OS: extend vlan.

- Theme: Trunking & Port Tagging:

  - Cisco IOS-XE: switchport mode trunk; switchport trunk allowed vlan [id].

  - Extreme EXOS: configure vlan [name] add ports [id] tagged.

  - Extreme VOSS: encapsulation dot1q; vlan members add [id] [port].

- Theme: Spanning Tree & Loop Prevention:

  - Cisco (Both): spanning-tree mode rapid-pvst; spanning-tree vlan [id] priority.

  - Extreme EXOS: enable stpd [name] auto-bind vlan; configure elrp-client.

  - Extreme VOSS: vlan create [id] type port-mstprstp [instance].

--------------------------------------------------------------------------------

Volume 6: [FAB-SDN] Fabric Connect & SD-Access

Deep-link mapping for software-defined overlay and underlay protocols.

- Theme: Control Plane Discovery & Adjacency:

  - Cisco SD-Access: router lisp; locator-table default; map-server.

  - Extreme Fabric Connect: router isis; spbm [id]; manual-area [id].

- Theme: Data Plane Encapsulation:

  - Cisco SD-Access: interface nve1; member vni [id].

  - Extreme Fabric Connect: vlan create [id] type spbm-bvlan; i-sid [id].

- Theme: Service Mapping (VNI to I-SID):

  - Cisco SD-Access: eid-table vrf [name]; database-mapping.

  - Extreme Fabric Connect: vlan i-sid [vlan-id] [isid-id]; auto-sense enable.

--------------------------------------------------------------------------------

Volume 7: [L3-VIRT] L3 Routing & Virtual Routing

Details IPv4/IPv6 address families and virtualized routing.

- Theme: Virtual Routing Instances (VRF):

  - Cisco IOS-XE: vrf definition [name]; address-family ipv4.

  - Extreme EXOS: create virtual-router [name]; configure vr [name] add ports.

  - Extreme VOSS: router vrf [name]; vrf-router configuration.

  - Extreme SLX-OS: show vrf.

- Theme: Dynamic Routing (OSPF/BGP):

  - Cisco IOS-XE: router ospf [id]; router bgp [as].

  - Cisco NX-OS: feature ospf; router ospf [tag].

  - Extreme EXOS: configure ospf add vlan [name] area [id]; configure bgp as-number.

  - Extreme VOSS: router ospf enable; router bgp enable; neighbor [ip] remote-as.

--------------------------------------------------------------------------------

Volume 8: [DIAG-LOG] Troubleshooting & Diagnostics

Maps the "Show/Clear/Debug" suites for real-time verification.

- Theme: Basic Connectivity & Path Tracing:

  - Cisco (Both): ping [ip]; traceroute [ip].

  - Extreme EXOS: ping [ip]; traceroute [ip].

  - Extreme VOSS: ping [ip]; traceroute [ip]; l2tracetree i-sid [id].

- Theme: Log Persistence & Event History:

  - Cisco IOS-XE: show logging; show platform software audit.

  - Cisco NX-OS: show logging log; show system internal [module] event-history.

  - Extreme EXOS: show log; debug hal [command].

  - Extreme VOSS: show logging; trace [level].

- Theme: Real-time Packet Capture:

  - Cisco (Both): ethanalyzer local interface [id]; show monitor capture buffer.

  - Extreme Switch: show ports [id] statistics.

--------------------------------------------------------------------------------

Volume 9: [MGMT-OPS] Operations & Session Management

Covers persistence, administrative labels, and session control.

- Theme: Configuration Persistence:

  - Cisco (Both): copy running-config startup-config; write memory.

  - Extreme EXOS: save; save configuration as-script.

  - Extreme VOSS: save config.

  - Extreme SLX-OS: copy running-config startup-config.

- Theme: Terminal & Session Parameters:

  - Cisco (Both): terminal length 0; show users.

  - Extreme EXOS: disable clipaging; show session.

  - Extreme VOSS: term more disable; show session.

- Theme: SNMP & Host Identification:

  - Cisco (Both): hostname [name]; snmp-server community.

  - Extreme EXOS: configure snmp sysName [name]; configure snmp add community.

  - Extreme VOSS: prompt [name]; snmp-server host [ip].