### Onboarding Mapping Table: ONBOARD Functional Group

This technical reference provides a multi-vendor command mapping for device onboarding, system initialization, and lifecycle management. As a Principal Network Systems Architect, I have curated these tables to prioritize model-driven standards (gNOI/gRPC) and precise CLI syntax derived strictly from the Cisco NX-OS Programmability Guide and the Extreme Switch Engine Command Reference.

#### 1. Onboarding and System Initialization Commands

The following table identifies the operations required for initial device restarts and the orchestration of the startup sequence.| Cisco IOS-XE | Cisco NX-OS | Extreme Switch Engine (EXOS) | Extreme Fabric Engine (VOSS) | SLX-OS || ------ | ------ | ------ | ------ | ------ || Information not available in source context | Operations managed via  gNOI (gRPC Network Operations Interface)  as the primary Operation Interface for system execution. | reboot {time month day year hour min sec} {cancel} {msmslot_id} {slotslot-number \| node-address \| stack-topology {as-standby}} | VSP Operating System Software (VOSS) was renamed to  Fabric Engine ; specific commands not available in source. | Information not available in source context |

#### 2. Configuration Reset and Factory Defaulting

This section maps the syntax for reversing configuration states. Per the source context, general factory defaulting is handled via specific "unconfigure" subsets in Switch Engine.| Cisco IOS-XE | Cisco NX-OS | Extreme Switch Engine (EXOS) | Extreme Fabric Engine (VOSS) | SLX-OS || ------ | ------ | ------ | ------ | ------ || Information not available in source context | State consistency is enforced via the  centralized DME database ; specific reset CLI not in source. | unconfigure ssl certificate trusted-ca \| ocsp-signature-ca file_name \| all | Information not available in source context | Information not available in source context |

#### 3. Boot Variable and Image Management

This mapping focuses on the selection of boot partitions and the programmatic rotation of system images.| Cisco IOS-XE | Cisco NX-OS | Extreme Switch Engine (EXOS) | Extreme Fabric Engine (VOSS) | SLX-OS || ------ | ------ | ------ | ------ | ------ || Information not available in source context | Software lifecycle and image rotation managed via  gNOI  and  iPXE  infrastructure. | download image; install image; use image primary \| secondary | Information not available in source context | Information not available in source context |

#### 4. Zero-Touch Provisioning (ZTP) and Auto-Configuration

These entries facilitate automated onboarding through programmatic agents or cloud-based discovery services.| Cisco IOS-XE | Cisco NX-OS | Extreme Switch Engine (EXOS) | Extreme Fabric Engine (VOSS) | SLX-OS || ------ | ------ | ------ | ------ | ------ || Information not available in source context | Utilizes  Model-Driven Programmability (MDP)  agents (NETCONF/RESTCONF/gRPC) and  iPXE  for automated state control. | configure auto-provision cloud-connector server; run provisioning; enable auto-provision cloud-connector | Information not available in source context | Information not available in source context |

#### 5. Command Negation and Disabling Syntax

The following table identifies the specific negation keywords required to deactivate onboarding features or cancel scheduled system operations.| Functional Action | OS Platform | Affirmative Command | Negation/Disable Command || ------ | ------ | ------ | ------ || System Reboot | Extreme Switch Engine | reboot | reboot cancel || Automated Provisioning | Extreme Switch Engine | enable auto-provision | disable auto-provision || Cloud Connectivity | Extreme Switch Engine | enable auto-provision cloud-connector | disable auto-provision cloud-connector || SSL Configuration | Extreme Switch Engine | configure ssl | unconfigure ssl |

#### 6. Operational Guidelines and CLI Conventions

The following architectural notes must be observed to ensure successful multi-vendor integration:

##### 6.1 Extreme Switch Engine CLI Syntax Conventions

Network architects must interpret CLI documentation using the following symbol standards defined in the Switch Engine Command Reference:

- Square Brackets   : Enclose a required value or a list of required arguments.

- Braces  { } : Enclose an optional value or a list of optional arguments.

- Vertical Bar  | : Separates mutually exclusive items in a list; one must be selected.

- Reserved XML Characters : The ampersand (&) and angle brackets (< and >) are strictly forbidden in naming conventions as they are reserved characters with special meaning in XML.

##### 6.2 Naming and Privilege Requirements

- Object Naming Rules : Names (e.g., VLANs) must begin with an alphabetical character, can contain underscores, and have a maximum length of 32 characters.

- Ambiguity Management : While names can be reused across categories (e.g., a VLAN and an STPD sharing a name), it is recommended to specify the  identifying keyword  (e.g., configure vlan name) to avoid system errors.

- Reserved Keywords : Specific keywords such as vlan or STP cannot be used as object names, though expanded forms (e.g., vlan10) are permissible.

- Access Control : Execution of initialization and configuration commands requires  administrator privilege  levels.

##### 6.3 Cisco NX-OS Model-Driven Infrastructure

- DME Database : Consistency across all interfaces (CLI, NETCONF, RESTCONF, gRPC) is maintained via a  centralized DME database . This ensures that the state of the switch remains uniform regardless of the management agent used.

- gNOI (gRPC Network Operations Interface) : In modern deployments, NX-OS leverages gNOI for operational tasks, including reboots and image management, moving beyond traditional CLI-based initialization.

- Platform Branding : Effective with Release 31.6,  ExtremeXOS (EXOS)  is officially  Switch Engine  and  VOSS  is officially  Fabric Engine .