# Network CLI Command Reference — Multi-Platform Cheat Sheet

IOS/IOS-XE  |  NX-OS  |  ExtremeXOS/VOSS  |  Aruba/ProCurve  |  JunOS  |  Arista EOS

## 1. System & General

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Show version | show version | show version | show version | show version | show version | show version |
| Show hostname | show running-config ¦ include hostname | show running-config ¦ include hostname | show switch | show system-information | show version ¦ match Hostname | show hostname |
| Set hostname | hostname <name> | hostname <name> | configure sysname <name> | hostname <name> | set system host-name <name> | hostname <name> |
| Show uptime | show version ¦ include uptime | show version ¦ include uptime | show switch | show system-information | show system uptime | show version ¦ grep uptime |
| Show CPU | show processes cpu | show processes cpu | show cpu-monitoring | show system-information | show chassis routing-engine | show processes top |
| Show memory | show processes memory | show processes memory | show memory | show system-information | show chassis routing-engine | show processes top |
| Save config | write memory copy run start | copy run start | save configuration | write memory | commit | write memory copy run start |
| Show running config | show running-config | show running-config | show configuration | show running-config | show configuration | show running-config |
| Show startup config | show startup-config | show startup-config | show configuration detail | show config | show configuration ¦ display set | show startup-config |
| Reload | reload | reload | reboot | reload | request system reboot | reload |

## 2. VLANs

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Create VLAN | vlan <id> name <name> | vlan <id> name <name> | create vlan <name> tag <id> | vlan <id> name <name> | set vlans <name> vlan-id <id> | vlan <id> name <name> |
| Show VLANs | show vlan | show vlan | show vlan | show vlans | show vlans | show vlan |
| Show VLAN detail | show vlan id <id> | show vlan id <id> | show vlan detail tag <id> | show vlans <id> detail | show vlans <name> | show vlan <id> |
| Access port | switchport mode access switchport access vlan <id> | switchport mode access switchport access vlan <id> | configure ports <p> vlan-members <id> untagged | vlan <id> untagged <port> | set interfaces <p> unit 0 family ethernet-switching vlan members <name> | switchport mode access switchport access vlan <id> |
| Trunk port | switchport mode trunk switchport trunk allowed vlan <id> | switchport mode trunk switchport trunk allowed vlan <id> | configure ports <p> vlan-members <id> tagged | vlan <id> tagged <port> | set interfaces <p> unit 0 family ethernet-switching trunk-mode | switchport mode trunk switchport trunk allowed vlan <id> |
| Native/untagged VLAN | switchport trunk native vlan <id> | switchport trunk native vlan <id> | configure ports <p> vlan-members <id> untagged | vlan <id> untagged <port> | set interfaces <p> native-vlan-id <id> | switchport trunk native vlan <id> |
| Show port VLAN | show interfaces <p> trunk show interfaces <p> switchport | show interfaces <p> trunk | show ports <p> vlan member | show vlans ports <p> | show interfaces <p> detail | show interfaces <p> trunk |

## 3. Interfaces

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Show interfaces | show interfaces | show interfaces | show ports | show interfaces | show interfaces | show interfaces |
| Show int status | show interfaces status | show interface status | show ports info | show interfaces status | show interfaces terse | show interfaces status |
| Show int brief | show ip interface brief | show ip interface brief | show ipconfig | show ip | show interfaces terse | show ip interface brief |
| Enable interface | no shutdown | no shutdown | enable port <p> | no shutdown | delete interfaces <p> disable | no shutdown |
| Disable interface | shutdown | shutdown | disable port <p> | shutdown | set interfaces <p> disable | shutdown |
| Set description | description <text> | description <text> | configure ports <p> display-string <text> | description <text> | set interfaces <p> description <text> | description <text> |
| Show errors | show interfaces <p> counters errors | show interfaces <p> counters errors | show ports <p> rxerrors show ports <p> txerrors | show interfaces <p> | show interfaces <p> detail ¦ match error | show interfaces <p> counters errors |
| Clear counters | clear counters <p> | clear counters interface <p> | clear counters ports <p> | clear statistics <p> | clear interfaces statistics <p> | clear counters <p> |
| Speed/duplex | speed <100¦1000¦auto> duplex <full¦auto> | speed <100¦1000¦auto> duplex <full¦auto> | configure ports <p> auto off speed 1000 duplex full | speed-duplex <mode> | set interfaces <p> ether-options speed <speed> | speed <speed> duplex full |

## 4. Spanning Tree

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Enable STP | spanning-tree mode <pvst¦rapid-pvst¦mst> | spanning-tree mode <mst¦rapid-pvst> | enable stpd <name> | spanning-tree | set spanning-tree interface <p> mode normal | spanning-tree mode <mode> |
| Show STP | show spanning-tree | show spanning-tree | show stpd | show spanning-tree | show spanning-tree bridge | show spanning-tree |
| Show STP detail | show spanning-tree detail | show spanning-tree detail | show stpd detail | show spanning-tree detail | show spanning-tree interface detail | show spanning-tree detail |
| PortFast/Edge | spanning-tree portfast | spanning-tree port type edge | configure stpd <n> ports edge-safeguard enable <p> | spanning-tree <p> admin-edge-port | set spanning-tree interface <p> edge true | spanning-tree portfast |
| BPDU Guard | spanning-tree bpduguard enable | spanning-tree bpduguard enable | configure stpd <n> ports edge-safeguard enable <p> | spanning-tree <p> bpdu-protection | set spanning-tree interface <p> no-root-port | spanning-tree bpduguard enable |
| Root guard | spanning-tree guard root | spanning-tree guard root | configure stpd <n> ports root-guard enable <p> | spanning-tree <p> root-guard | set spanning-tree interface <p> no-root-port | spanning-tree guard root |
| Set root bridge | spanning-tree vlan <id> root primary | spanning-tree vlan <id> root primary | configure stpd <n> priority 4096 | spanning-tree priority 0 | set spanning-tree bridge priority 4096 | spanning-tree vlan <id> priority 4096 |

## 5. Routing

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Show routes | show ip route | show ip route | show iproute | show ip route | show route | show ip route |
| Show route detail | show ip route <prefix> | show ip route <prefix> | show iproute <prefix> detail | show ip route <prefix> | show route <prefix> detail | show ip route <prefix> |
| Default route | ip route 0.0.0.0 0.0.0.0 <nh> | ip route 0.0.0.0/0 <nh> | configure iproute add default <nh> | ip route 0.0.0.0 0.0.0.0 <nh> | set routing-options static route 0.0.0.0/0 next-hop <nh> | ip route 0.0.0.0/0 <nh> |
| Static route | ip route <prefix> <mask> <nh> | ip route <prefix>/<len> <nh> | configure iproute add <prefix>/<len> <nh> | ip route <prefix> <mask> <nh> | set routing-options static route <prefix>/<len> next-hop <nh> | ip route <prefix>/<len> <nh> |
| Show OSPF neighbors | show ip ospf neighbor | show ip ospf neighbors | show ospf neighbor | show ip ospf neighbor | show ospf neighbor | show ip ospf neighbor |
| Show BGP summary | show ip bgp summary | show bgp summary | show bgp | show bgp summary | show bgp summary | show ip bgp summary |
| Ping | ping <ip> | ping <ip> | ping <ip> | ping <ip> | ping <ip> | ping <ip> |
| Traceroute | traceroute <ip> | traceroute <ip> | traceroute <ip> | traceroute <ip> | traceroute <ip> | traceroute <ip> |

## 6. LLDP / CDP

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Enable LLDP | lldp run | feature lldp | enable lldp ports all | lldp run | set protocols lldp interface all | lldp run |
| Show LLDP neighbors | show lldp neighbors | show lldp neighbors | show lldp neighbors | show lldp neighbors | show lldp neighbors | show lldp neighbors |
| Show LLDP detail | show lldp neighbors detail | show lldp neighbors detail | show lldp neighbors detail | show lldp neighbors detail | show lldp neighbors detail | show lldp neighbors detail |
| Show CDP neighbors | show cdp neighbors | show cdp neighbors | N/A | N/A | N/A | N/A |

## 7. ACLs / Firewall Filters

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Create ACL | ip access-list extended <name> | ip access-list <name> | create acl <name> type L3 | ip access-list extended <name> | set firewall filter <name> | ip access-list <name> |
| Permit statement | permit ip <src> <dst> | permit ip <src> <dst> | entry <n> { if { source-address <src>; } then { permit; } } | permit ip <src> <dst> | term <n> { from { source-address <src>; } then { accept; } } | permit ip <src> <dst> |
| Deny statement | deny ip <src> <dst> | deny ip <src> <dst> | entry <n> { if { source-address <src>; } then { deny; } } | deny ip <src> <dst> | term <n> { from { source-address <src>; } then { reject; } } | deny ip <src> <dst> |
| Apply to interface | ip access-group <name> in | ip access-group <name> in | configure acl <name> ports <p> ingress | ip access-group <name> in | set interfaces <p> unit 0 family inet filter input <name> | ip access-group <name> in |
| Show ACLs | show access-lists | show ip access-lists | show acl | show access-list | show firewall filter | show ip access-lists |

## 8. MAC Address Table

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Show MAC table | show mac address-table | show mac address-table | show fdb | show mac-address | show ethernet-switching table | show mac address-table |
| Show MAC for port | show mac address-table interface <p> | show mac address-table interface <p> | show fdb ports <p> | show mac-address port <p> | show ethernet-switching interface <p> | show mac address-table interface <p> |
| Show MAC for VLAN | show mac address-table vlan <id> | show mac address-table vlan <id> | show fdb vlan <name> | show mac-address vlan <id> | show ethernet-switching table vlan <name> | show mac address-table vlan <id> |
| Clear MAC table | clear mac address-table dynamic | clear mac address-table dynamic | clear fdb | clear mac-address | clear ethernet-switching table | clear mac address-table dynamic |

## 9. Logging, Syslog & NTP

| Function | IOS/IOS-XE | NX-OS | XOS/VOSS | Aruba/ ProCurve | JunOS | Arista EOS |
| --- | --- | --- | --- | --- | --- | --- |
| Show logs | show logging | show logging last 100 | show log | show logging | show log messages | show logging |
| Set syslog server | logging host <ip> | logging server <ip> | configure syslog add <ip> | logging <ip> | set system syslog host <ip> any notice | logging host <ip> |
| Set log level | logging trap <level> | logging level <facility> <level> | configure syslog add <ip> severity <level> | logging severity <level> | set system syslog host <ip> any <level> | logging trap <level> |
| Show NTP status | show ntp status show ntp associations | show ntp peer-status | show ntp | show ntp associations | show ntp associations | show ntp associations |
| Set NTP server | ntp server <ip> | ntp server <ip> | configure ntp add <ip> | ntp server <ip> | set system ntp server <ip> | ntp server <ip> |